ROS 2 Assignment

Robotic Arm URDF, Simulation & Serial Communication

Robotics Course  |  Two-Week Project

📌 Overview This assignment is split into two parts with separate submission deadlines: • Part 1 — URDF & Simulation: due next week • Part 2 — Arduino Serial Communication: due the week after The goal is to model a robotic arm, visualize it in a simulator, publish joint trajectories over ROS 2, and finally bridge those commands to real servo motors via an Arduino.

PART 1 — URDF & Simulation

Submission deadline: Next week

## 1. The Arm You Are Modelling

The diagram below shows the arm's physical structure and dimensions. Use these measurements to set the size of your URDF links. Each link should be modelled as a simple box or cylinder — no meshes required (You can add meshes if you want).

Figure 1 — Robotic arm dimensions (Side view)

📐 Link Dimensions Summary Base: The height from the working plan is 7.1 cm. L1 — 10 cm tall. L2 — 12.8 cm long segment connecting L1 to L3. L3 — 7 cm long segment leading to the end effector. End-effector (Gripper) — 11 cm from L3 to the end effector tip.

## 2. Key Definitions

Before you start writing URDF, make sure you understand these fundamental concepts. They will appear in every robotics project you work on.

## 2.1 URDF

URDF (Unified Robot Description Format) is an XML-based file format used in ROS to describe the physical structure of a robot. Think of it as a blueprint that tells the simulator, visualization tools, and controllers exactly what the robot looks like and how its parts are connected.

Before you start writing URDF, make sure you understand these fundamental concepts. They will appear in every robotics project you work on.

link → joint → link → joint → link → … → end_effector link

| Term | Definition |
| --- | --- |
| Link | A link is a single rigid body — one physical part of the robot. It can be a base plate, an arm segment, a gripper finger, or anything that does not bend or flex on its own. In URDF, each link has a visual shape (what it looks like), a collision shape (used for physics), and inertial properties (mass and inertia). |
| Joint | A joint connects two links together and defines the motion allowed between them. A revolute joint rotates around a single axis (like a servo motor), a prismatic joint slides along an axis, and a fixed joint means the two links cannot move relative to each other. Every joint has a parent link and a child link. (Each joint in your URDF corresponds to a motor or servo in reality.) |
| End Effector | The end effector is the "business end" of the arm — the point that actually interacts with the world. On a gripper it is the tip where the fingers close. On a drill it is the drill bit tip. On a welding arm it is the torch nozzle. In URDF it is usually represented as a small fixed link at the very end of the kinematic chain. |
| Base Link | The root link of the robot — the one that is fixed to the ground or a mounting surface. All other links are children of this one, directly or indirectly. In the arm above, (Base) acts as the base. |
| Kinematic Chain | The sequence of links and joints from the base to the end effector. For this arm the chain is: base_link → joint1 → L1 → joint2 → L2 → joint3 → L3 → joint4 → end_effector. |

### Joint Types

Choose the joint type that matches the physical joint of your arm:

| Type | Meaning |
| --- | --- |
| revolute | Rotates around one axis, with defined min/max angle limits. Used for servo joints. |
| continuous | Rotates around one axis with no limits (like a wheel). |
| prismatic | Slides along an axis (linear motion). |
| fixed | No motion — used to rigidly attach sensors, end-effectors, etc. |

## 3. Simulating using Gazebo (gz sim)

## Gazebo (gz sim) is the recommended modern simulator for ROS 2 Jazzy. To load your arm, you need a world SDF file that spawns the URDF, and the ros_gz_bridge package to bridge topics.

### Resources

The official ROS2 Jazzy has a great tutorial of how to make a URDF file and visualize it using Rviz

https://docs.ros.org/en/jazzy/Tutorials/Intermediate/URDF/Building-a-Visual-Robot-Model-with-URDF-from-Scratch.html#

And for Gz sim you can see the official tutorial

https://gazebosim.org/docs/jetty/tutorials/

## 4. Control the arm

### 4.1 Understanding trajectory_msgs/msg/JointTrajectory

This message type lets you command multiple joints simultaneously. The key fields are:

JointTrajectory Message Structure joint_names — A list of joint name strings (must match your URDF joint names exactly). points — A list of trajectory points. Each point contains: positions — Joint angles in radians (one per joint in joint_names). time_from_start — When to reach this point (sec + nanosec). velocities, accelerations, effort — Not needed for servos; leave empty.

💡 Example Message (CLI) The message below commands joint1 to 1.0 rad and joint2 to 0.5 rad: joint_names: ['joint1', 'joint2'] points: - positions: [1.0, 0.5] time_from_start: sec: 2 nanosec: 0

## 5. Part 1 — Deliverables

Checklist □ URDF file with at least 4 revolute joints (base + 3 arm segments minimum) □ All links have <visual>, <collision>, and <inertial> □ Robot displays correctly in RViz2 □ Robot loads and is visible in Gazebo (gz sim) □ Python publisher node successfully publishes to /arm_controller/joint_trajectory and arm moving in gz sim

PART 2 — Arduino Serial Communication

Submission deadline: The week after Part 1

🔗 System Architecture ROS 2 PC ──[ USB / Serial ]── Arduino ──[ Signal ]── Servo Motors 1. A ROS 2 node subscribes to /arm_controller/joint_trajectory. 2. On receiving a message, it formats angles into a compact ASCII string and sends it over USB. 3. The Arduino reads that string from Serial, parses the joint angles, and drives servos via PWM.

## 1. ROS 2 Serial Bridge Node

This node bridges ROS 2 and the Arduino. It subscribes to the joint trajectory topic and writes angle data to the serial port.

### 1.1 Install pyserial

pip3 install pyserial # Also add it to your package.xml: # <exec_depend>python3-serial</exec_depend>

### 1.2 Serial Bridge Node

Create serial_bridge.py:

This ROS2 code will be responsible for connecting to the arduino through USB, Subscribe from /arm_controller/joint_trajectory send the joint angles to the arduino.

📦 Serial Protocol Format sent by ROS 2 to Arduino: <angle1>,<angle2>,<angle3>,<angle4>,<angle5>\n Example: 90,45,120,60,0\n Angles are in degrees (0–180), already clamped to servo range. The \n newline character acts as the message terminator the Arduino waits for.

### Note

You can make the message structure as you like because you will be also the one responsible for making an arduino code to parse it back to angles.

Angle 5: is just a joint responsible for opening and closing the gripper

## 2. Arduino Code

The Arduino code will have to read the serial message, parse the 5 angles, and command the servos.

## 3. Wiring Reference

There will be 5 servos in total one for rotating the base one for moving L1-L2, L2-L3, L3-Gripper and final one to open and close the gripper

| Arduino Pin | Connects to |
| --- | --- |
| - | Servo 1 signal wire (base rotation) |
| - | Servo 2 signal wire (L1-L2) |
| - | Servo 3 signal wire (L2-L3) |
| - | Servo 4 signal wire (L3-Gripper) |
| - | Servo 5 open and close the gripper |
| USB | Connected to PC running ROS 2 (appears as /dev/ttyUSB0 or /dev/ttyACM0) |

## 4. Testing the Full Pipeline

You have to make a tool or use any existing tool so that you can send commands via Ros2 to change what /arm_controller/joint_trajectory publishes manually.

## 5. Part 2 — Deliverables

Checklist □ serial_bridge.py ROS 2 node compiles and runs without errors □ Arduino sketch uploaded and servos respond to serial commands □ Full pipeline demo: Tool (GUI) → /arm_controller/joint_trajectory → serial → servos move

IMPORTANT NOTE!

Usually, when learning robotics, we start by using off-the-shelf models, robots, or robotic arms. However, due to the lack of time, we may not have the opportunity to work with a ready-to-use robotic arm during the course. Nevertheless, that does not prevent you from experimenting with one later.

For example, you can explore platforms such as TurtleBot 4, which is a popular open-source robotics platform for education and research that comes with ROS 2 support and built-in sensors like LiDAR and cameras.

Some useful resources include:
https://github.com/turtlebot/turtlebot4
https://github.com/UniversalRobots/Universal_Robots_ROS2_Driver

"Moving a real servo from a ROS 2 topic — that is the moment simulation meets reality."